HO CHI MINH UNIVERSITY OF TECHNOLOGY AND EDUCATION

FACULTY OF INTERNATIONAL EDUCATION

FINAL REPORT: MACHINE VISION

INSTRUCTOR: DR. BUI HA DUC

COURSE: MAVI332529E

STUDENTS’ NAME AND ID

NGUYEN DANG KHOA 		23146015

LE TAN PHAT                                    23146024

Subject: Badminton Player Detection, Tracking and Heatmap Analysis

HO CHI MINH CITY, JUNE 2025

# 1. Executive Summary

This project details the development of an automated computer vision pipeline for generating badminton player activity heatmaps. The system addresses the challenges of oblique camera angles and complex motion segmentation without relying on heavy deep-learning frameworks, offering a transparent, mathematically rigorous solution for sports analytics.

# 2. Introduction

In the context of modern sports science, data-driven analysis is crucial for performance optimization. While many existing solutions utilize deep learning detection models, this project demonstrates the effectiveness of classical computer vision techniques. By combining geometric transformation, background modeling, and global optimization tracking, we have constructed a system capable of accurately mapping player movement on a regulation badminton court.

# 3. Methodological Pipeline

## 3.1 Coordinate System & Geometric Projection

The primary technical hurdle is that broadcast cameras are rarely perpendicular to the court surface. We rectify this using a Perspective Transformer class. By selecting four non-collinear points, the system computes the homography matrix M to map pixel coordinates into a real-world metric plane (6.1m x 13.4m). This projection is essential for ensuring that heatmap data reflects physical court coverage rather than relative image position.

## 3.2 Adaptive Background Subtraction (MOG2)

Foreground segmentation is handled by a Mixture of Gaussians (MOG2) background subtractor. Unlike simpler models, MOG2 adapts to dynamic lighting environments. We refine the output mask through:

- • Morphological Filtering: Using elliptic kernels to perform opening and closing operations, which successfully separate players from the court surface while removing non-human noise.

- • ROI Masking: Applying a region-of-interest mask to eliminate extraneous objects outside the court boundary.

## 3.3 Global Optimization Tracking

To ensure robust player tracking, we implemented a system using the Hungarian Algorithm (`linear_sum_assignment`). This global approach solves the assignment problem by minimizing the cost function of IoU overlap and center-distance displacement. This mechanism significantly outperforms greedy matching, maintaining identity persistence even during intense rally exchanges.

# 4. Advanced Features: Near-Net Mode & Sticky Assignment

A core innovation in this version is the 'Near-Net' tracking mode. When the tracker identifies that two players are in close proximity near the net—a common scenario where traditional IoU-based tracking might fail due to bounding box overlap—it automatically adjusts its distance thresholds. Complementing this, the 'Sticky Slot Assigner' uses the court's mid-line as a logical boundary to maintain a permanent ID slot (e.g., 'Top' vs 'Bottom' player) for each athlete, eliminating ID-flickering and providing consistent per-player heatmap data.

# 5. Implementation Details & Configuration

The system relies on a unified configuration structure, allowing researchers to fine-tune parameters for various camera perspectives and court lighting conditions:

- • Spatial Settings: `HEATMAP_RES_M` defines the grid granularity.

- • Detector Sensitivity: `MIN_AREA` and `MAX_AREA` constraints prevent false detections.

- • Optimization Weightings: Costs are balanced between IoU, centroid distance, and aspect ratio similarity to prioritize matching accuracy.

# 6. Discussion: Results & Visualization

The output consists of a timestamped CSV file and a series of activity heatmaps. The Gaussian smoothing filter is applied post-accumulation to ensure that the heatmap represents meaningful movement 'intensity' rather than sparse point data. These visualizations offer coaches and players clear insights into court utilization, revealing patterns such as defensive positioning, movement efficiency, and service-zone preferences.

# 7. Conclusion

This project validates the feasibility of building high-performance sports analytics tools using fundamental computer vision techniques. By focusing on geometric stability and global identity assignment, we have developed a tool that is not only robust but also highly interpretable and customizable for different sporting environments